PRO-Series Direct Entry Programmable Scanners

General Guide To Scanning                             Faxback Doc. # 17653

Birdies

Birdies are frequencies your scanner uses when it operates.  These

operating frequencies might interfere with broadcasts on the same

frequencies.  If you program one of these frequencies, you hear only noise

on that frequency.

If the interference is not severe, you might be able to turn SQUELCH

clockwise to cut out the birdie.  The most common birdies to watch for are

listed below.

Birdie Frequencies:

31.05 MHz             124.20 MHz

41.40 MHz             134.55 MHz

51.75 MHz             144.90 MHz

113.85 MHz            155.25 MHz

Reception Notes

Reception of the frequencies covered by your scanner is mainly "line of

sight".  That means you usually cannot hear stations that are beyond the

horizon.  During the summer months you may be able to hear stations in the

30-50 MHz range located several hundred or even thousand of miles away.

This is because of summer atmospheric conditions.  This type of reception

is unpredictable but often very interesting!

One very useful service is the National Weather Service's continuous

weather broadcast.  These broadcasts contain weather forecasts and data

for the areas around the station, plus bulletins on any threatening

weather conditions.  These stations use three frequencies - 162.40,

162.475 or 162.55 MHz.  In most areas of the country, you can receive one

of these frequencies.

A Guide To The Action Bands

With the right frequencies programmed into your PRO-Series Scanner, you

can monitor exciting events.  With a little investigation, you can find

active frequencies in your community.  We can give you some general

pointers, and you can take it from there.  Please use caution and common

sense when you hear an emergency call.  Never go to the scene of an

emergency.  It could be very dangerous.

Find out if there is a local club that monitors your community's

frequencies.  Perhaps a local electronics repair shop that works on

equipment similar to your scanner can give you frequencies used by local

radio services.

A volunteer police department or fire department can also be a good source

for this information.

As a general rule on VHF, most activity is concentrated between 153.785

and 155.98 MHz and then again from 158.73 to 159.46 MHz.  Here you find

local government, police, fire and most such emergency services.  If you

are near a railroad yard or major railroad tracks, look around 160.0 to

161.9 MHz for signals.

In some larger cities, there has been a move to the UHF bands for

emergency service.  Here, most of the activity is between 453.025 and

453.95 MHz and between 456.025 and 467.925 MHz.

In the UHF band, frequencies between 456.025 and 459.95 MHz and between

465.025 and 469.975 MHz are used by mobile units and control stations

associated with base and repeater units that operate 5 MHz lower (that is,

451.025 to 454.950 and 460.025 to 464.975 MHz).  This means that if you

find an active frequency inside one of these spreads, you can look 5 MHz

lower (or higher) to find the base station/repeater for that service.

Typical Band Usage

The following is a brief listing of the typical services that use the

bands you scanner can receive.  This listing helps you decide which ranges

you would like to scan.

These frequencies are subject to change, and might vary from area to area.

For a more complete listing refer to the Police Call Radio Guide available

at you local RadioShack store.

Abbreviations:

Affiliate Radio System: ............................................. Mars

Amateur: ............................................................. Ham

Automobile Emergency: ......................................... Auto Emer.

Broadcast Remote: ................................................... BC.R

Bureau of Reclamation: ......................................... Bur.Recl.

Civil Air Patrol: .................................................... CAP

Department of Agriculture and Forestry: .................... Agr. And For.

Fire Department: .................................................... F.D.

Forest Products: ............................................... For.Prod.

Forestry Conservation: ........................................ Fors.Cons.

Government: ........................................................ Govt.

Highway Maintenance: ................................................ Hwy.

Land Transportation: ............................................ Land Tr.

Local Government: ................................................ L.Govt.

Manufacturers: ...................................................... Mfg.

Military: ............................................................ MIL

Mobile Telephone : .............................................. Mob.Tel.

Motion Picture: ................................................... Mot.P.

Motor Carrier: .............................................. Buses.Trucks

National Parks: ................................................. Nat.Park

Petroleum: .......................................................... Pet.

Police: ............................................................. P.D.

Power Utilities: ................................................... Power

Radio Paging: ....................................................... Page

Railroad: ........................................................... R.R.

Relay Press: ....................................................... Press

State Police: .................................................... St.P.D.

Special Emergency: .............................................. Sp.Emer.

Special Industry: ................................................ Sp.Ind.

Taxicab Radio: ...................................................... Taxi

Telephone Maintenance: ........................................ Tel.Maint.

U.S. Coastal and Geodetic Survey: ............................. U.S.C.G.S.

U.S. Navy: ........................................................... USN

U.S. Weather Bureau: ............................................ U.S.W.B.

ATTENTION:  Your scanner may not be able to receive all frequencies and/or

modes of reception that are contained within this document.

For complete information of your scanner's capabilities, be

sure to read your owner's manual completely.

Guide To Frequencies

National Weather Frequencies:

1) 161.650              5) 162.440              9) 162.525

2) 161.775              6) 162.450             10) 162.550

3) 162.400              7) 162.475             11) 163.275

4) 162.425              8) 162.500

Ham Radio Frequencies

Ham operators often transmit emergency information when other

communication methods break down.  The following chart shows some of the

frequencies that Hams use.

Wavelength                        Frequency

(Meters)                            (MHz)

10-meter                        28.000-29.700

6-meter                         50.000-54.000

2-meter                        144.000-148.000

70-cm                          420.000-450.000

The following are the channels and frequencies of the Citizens Band:

1) 26.965     21) 27.215

2) 26.975     22) 27.225

3) 26.985     23) 27.255

4) 27.005     24) 27.235

5) 27.015     25) 27.245

6) 27.025     26) 27.265

7) 27.035     27) 27.275

8) 27.055     28) 27.285

9) 27.065     29) 27.295

10) 27.075     30) 27.305

11) 27.085     31) 27.315

12) 27.105     32) 27.325

13) 27.115     33) 27.335

14) 27.125     34) 27.345

15) 27.135     35) 27.355

16) 27.155     36) 27.365

17) 27.165     37) 27.375

18) 27.175     38) 27.385

19) 27.185     39) 27.395

20) 27.205     40) 27.405

Guide To The Action Bands

United States Broadcast Bands

In the United States, there are several broadcast bands. The standard AM

and FM bands are probably the most well known.  There are also four

television audio broadcast bands-the lower three transmit on the VHF band

and the fourth transmits on the UHF band.

Frequency Range                                             Allocation

54.0 - 72.0 MHz ........................................... VHF Television

76.0 - 88.0 MHz ........................................... VHF Television

88.0 - 108.0 MHz ............................................. Standard FM

174.0 - 216.0 MHz ......................................... VHF Television

470.0 - 805.75 MHz ........................................ UHF Television

International Broadcast Bands

Several short-wave bands are allocated for international broadcasting

because of the nature of propagation of high frequencies.  The bands are

sometimes identified according to the approximate wavelength of the

signals in meters.  Your scanner may receive the 11-meter band, from

25.6 - 26.10 MHz.

Typical Band Usage

HF Band (3.0 - 30.0 MHz):

Mid Range: ............................................. 25.00 - 28.63 MHz

10-Meter Amateur Band: ................................. 28.00 - 29.70 MHz

High Range: ............................................ 29.70 - 29.90 MHz

VHF Band (30.00 - 300.0 MHz):

Low range: ............................................. 30.00 - 50.00 MHz

6-Meter Amateur: ....................................... 50.00 - 54.00 MHz

FM-TV Audio Broadcast, Wide Band: ...................... 54.00 - 72.00 MHz

FM Radio Broadcast, Wide Band: ........................ 88.00 - 108.00 MHz

Aircraft: ............................................ 108.00 - 136.00 MHz

U.S. Government: ..................................... 138.00 - 144.00 MHz

2-Meter Amateur: ..................................... 144.00 - 148.00 MHz

High Range: .......................................... 148.00 - 174.00 MHz

New Mobile Narrow Band: .............................. 220.00 - 222.00 MHz

1.3-Meter Amateur: ................................... 222.00 - 225.00 MHz

Military Aircraft: ................................... 225.00 - 287.80 MHz

UHF Band (300.00 MHz - 3.0 GHz):

Military Aircraft: ................................... 311.00 - 384.00 MHz

U.S. Government: ..................................... 406.00 - 470.00 MHz

0.6-Meter Amateur: ................................... 420.00 - 450.00 MHz

Low Range: ........................................... 450.00 - 470.00 MHz

FM-TV Audio Broadcast, Wide Band: .................... 470.00 - 806.00 MHz

Conventional Systems: ................................ 851.00 - 856.00 MHz

Conventional/Trunked Systems: ........................ 856.00 - 861.00 MHz

Trunked Systems: ..................................... 861.00 - 866.00 MHz

Public Safety: ....................................... 866.00 - 869.00 MHz

Common Carrier: ...................................... 869.00 - 894.00 MHz

Private Trunked: ..................................... 935.00 - 940.00 MHz

General Trunked: ..................................... 940.00 - 941.00 MHz

Primary Usage:

As a general rule, most of the radio activity is concentrated on the

following frequencies:

VHF Band:

2-Meter Amateur Band: .............................. 144.000 - 148.000 MHz

Government, police, and Fire: ...................... 153.785 - 155.980 MHz

Emergency Services: ................................ 158.730 - 159.460 MHz

Railroad: .......................................... 160.000 - 161.900 MHz

UHF Band:

.6 cm Amateur Band FM Repeaters: ................... 440.000 - 450.000 MHz

Land Mobile "Paired" Frequencies: .................. 450.000 - 470.000 MHz

Base Stations: ..................................... 451.025 - 454.950 MHz

Mobile Units: ...................................... 456.025 - 459.950 MHz

Repeater Units: .................................... 460.025 - 464.975 MHz

Control Stations: .................................. 465.025 - 469.975 MHz

NOTE:  UHF remote control stations and mobile units typically operate at 5

MHz higher than their associated base and relay repeater units.

Specified Intervals

Frequencies in different bands are accessible only at specific intervals.

For Example:

VHF, HAM, and Government: .................................. 5.0 kHz steps

All Others: ............................................... 12.5 kHz steps

Aircraft: ................................................. 25.0 kHz steps

Note:  Your scanner rounds the entered frequency to the nearest valid

frequency. For example, if you try to enter 151.473, the scanner

might accept this as 151.470.

Band Allocation

To help you decide which frequency ranges to search, use the following

listing of the typical services that use the frequencies your scanner

receives. These frequencies are subject to change, and might vary from

area to area. For a more complete listing, refer to the "Police-Call Radio

Guide including Fire and Emergency Services", as well as "Beyond Police

Call", "Aeronautical Directory", "Nautical Directory" and "Now you're

Talking" texts available at your local RadioShack store.

Abbreviations

AIR: ............................................................ Aircraft

BIFC: .................................. Boise (ID) Interagency Fire Cache

BUS: ............................................................ Business

CAP: .................................................... Civil Air Patrol

CB: ........................................................ Citizens Band

CCA: ...................................................... Common Carrier

CSB: ................................................ Conventional Systems

CTSB: ....................................... Conventional/Trunked Systems

FIRE: .................................................... Fire Department

HAM: ................................................. Amateur (HAM) Radio

GOVT: ................................................. Federal Government

GMR: ................................................ General Mobile Radio

GTR: ..................................................... General Trunked

IND: ................................................. Industrial Services

MARI: ............................................. Maritime Limited Coast

MARS: .................................... Military Affiliate Radio System

MED: .......................................... Emergency/Medical Services

MIL: ....................................................... U.S. Military

MOV: ....................................... Motion Picture/Video Industry

NEW: ................................................... New Mobile Narrow

NEWS: ........................................................ Relay Press

OIL: .............................................. Oil/Petroleum Industry

POL: ................................................... Police Department

PUB: ..................................................... Public Services

PSB: ....................................................... Public Safety

PTR: ..................................................... Private Trunked

ROAD: ......................................... Road & Highway Maintenance

RTV: .................................... Radio/TV Remote Broadcast Pickup

TAXI: ...................................................... Taxi Services

TELBL: .................................................. Mobile Telephone

TELC: ................................................ Cordless Telephones

TELM: .............................................. Telephone Maintenance

TOW: .......................................................... Tow Trucks

TRAN: ............................................ Transportation Services

TSB: ..................................................... Trunked Systems

TVn: ............................................... FM-TV Audio Broadcast

USXX: .............................................. Government Classified

UTIL: ............................................ Power & Water Utilities

WTHR: ............................................................ Weather

High Frequency (HF)-(3 - 30 MHz):

High Band - (25.00 - 27.36 MHz):

25.020 - 25.320: ..................................................... IND

25.870 - 26.470: ..................................................... RTV

26.62: ............................................................... CAP

26.966 - 27.405: ...................................................... CB

27.430 - 27.630: ..................................................... BUS

10-Meter Amateur Band - (28.0 - 29.7 MHz):

28.000 - 29.700: ..................................................... HAM

Very High Frequency (VHF) - (30 - 300 MHz):

Low Band - (29.7 - 50 MHz - in 5 kHz steps):

29.700 - 29.790: ..................................................... IND

29.900 - 30.550: ............................................... GOVT, MIL

30.580 - 31.980: ................................................ IND, PUB

32.000 - 32.990: ............................................... GOVT, MIL

33.020 - 33.980: ........................................... BUS, IND, PUB

34.010 - 34.990: ............................................... GOVT, MIL

35.020 - 35.980: ..................................... BUS, PUB, IND, TELM

36.000 - 36.230: ............................................... GOVT, MIL

36.250: ............................................... Oil spill clean up

36.270 - 36.990: ............................................... GOVT, MIL

37.020 - 37.980: ................................................ PUB, IND

38.000 - 39.000: ............................................... GOVT, MIL

39.020 - 39.980: ..................................................... PUB

40.000 - 42.000: ......................................... GOVT, MIL, MARI

42.020 - 42.940: ..................................................... POL

42.960 - 43.180: ..................................................... IND

43.220 - 43.680: .......................................... TELM, IND, PUB

43.700 - 44.600: .................................................... TRAN

44.620 - 46.580: ................................................ POL, PUB

46.600 - 46.990: .............................................. GOVT, TELC

47.020 - 47.400: ..................................................... PUB

47.420: ............................................... American Red Cross

47.440 - 49.580: ................................................ IND, PUB

49.610 - 49.990: ............................................... MIL, TELC

6-METER Amateur Band (50-54 MHz):

50.00 - 54.00: ....................................................... HAM

FM-TV Audio Broadcast, Wide Band (54-72 MHz):

59.750: .............................................................. TV2

65.750: .............................................................. TV3

71.750: .............................................................. TV4

Land Mobile Service Band (72-76 MHz):

FM-TV Audio Broadcast, Wide Band (76-88 MHz):

81.750: .............................................................. TV5

87.750: .............................................................. TV6

FM Radio Broadcast, Wide Band (88-108 MHz):

Aircraft Band (108-136 MHz):

108.000 - 121.490: ................................................... AIR

121.500: ................................................... AIR emergency

121.510 - 136.000: ................................................... AIR

U.S. GOVERNMENT BAND (138-144 MHz):

137.000 - 144.000: ............................................. GOVT, MIL

VHF-Hi BAND (148-174 MHz):

148.050 - 150.345: ........................................ CAP, MARS, MIL

150.775 - 150.790: ................................................... MED

150.815 - 150.965: ................................................... TOW

150.980: .............................................. Oil spill clean up

150.995 - 151.130: .................................................. ROAD

151.145 - 151.475: ................................................... POL

151.490 - 151.955: .............................................. IND, BUS

151.985: ............................................................ TELM

152.030 - 152.240: .................................................. TELB

152.270 - 152.465: ............................................. IND, TAXI

152.480: ............................................................. BUS

152.510 - 152.840: .................................................. TELB

152.870 - 153.020: .............................................. IND, MOV

153.035 - 153.175: ........................................ IND, OIL, UTIL

153.740 - 154.445: ............................................. PUB, FIRE

154.490 - 154.570: .............................................. IND, BUS

154.585: .............................................. Oil spill clean up

154.600 - 154.625: ................................................... BUS

154.665 - 156.240: ................................... MED, ROAD, POL, PUB

165.255: ............................................................. OIL

156.275 - 157.425: .................................................. MARI

157.450: ............................................................. MED

157.470 - 157.515: ................................................... TOW

157.530 - 157.725: ............................................. IND, TAXI

157.740: ............................................................. BUS

157.770 - 158.100: .................................................. TELB

158.130 - 158.460: ............................. BUS, IND, OIL, TELM, UTIL

158.490 - 158.700: .. ............................................... TELB

158.730 - 159.465: ........................................ POL, PUB, ROAD

159.480: ............................................................. OIL

159.495 - 161.565: .................................................. TRAN

161.580: ............................................................. OIL

161.600 - 162.000: ............................................. MARI, RTV

162.0125 - 162.35: ....................................... GOVT, MIL, USXX

162.400 - 162.550: .................................................. WTHR

162.5625 - 162.6375: ..................................... GOVT, MIL, USXX

162.6625: ............................................................ MED

162.6875 - 163.225: ...................................... GOVT, MIL, USXX

163.250: ............................................................. MED

163.275 - 166.225: ....................................... GOVT, MIL, USXX

166.250: ................................................. GOVT, RTV, FIRE

166.275 - 169.400: ............................................ GOVT, BIFC

169.445: .................................................. WIRELESS MIKES

169.500: ............................................................ GOVT

169.505: .................................................. WIRELESS MIKES

169.55 - 169.9875: ....................................... GOVT, MIL, USXX

170.000: ............................................................ BIFC

170.025 - 170.150: ....................................... GOVT, RTV, FIRE

170.175 - 170.225: .................................................. GOVT

170.245 - 170.305: ........................................ WIRELESS MIKES

170.350 - 170.400: ............................................. GOVT, MIL

170.425 - 170.450: .................................................. BIFC

170.475: ............................................................. PUB

170.4875 - 173.175: ............................ GOVT, PUB, WIRELESS MIKES

173.225 - 173.375: ....................................... MOV, NEWS, UTIL

173.3875 - 178.5375: ................................................. MIL

173.5625 - 173.5875: ............................. MIL Medical/Crash Crews

173.60 - 173.9875: .................................................. GOVT

FM-TV Audio Broadcast, VHF Wide Band (174-216 MHz):

179.750: ............................................................. TV7

185.750: ............................................................. TV8

191.750: ............................................................. TV9

197.750: ............................................................ TV10

203.750: ............................................................ TV11

209.750: ............................................................ TV12

215.750: ............................................................ TV13

New Mobil Narrow Band (220-222 MHz):

220.000 - 222.000: ................................................... NEW

1.3-Meter Amateur Band (222-225 MHz):

222.000 - 225.000: ................................................... HAM

MILITARY AIRCRAFT BAND (237.9-287.8 MHz):

237.900: ..................................... Coast Guard Search & Rescue

239.800: ..................................................... FAA Weather

241.000: ............................................................ ARMY

243.000: ....................................................... EMERGENCY

255.400: .............................................. FAA FLIGHT SERVICE

257.800: ................................................. CIVILIAN TOWERS

287.800: ............................................. Coast Guard Air/Sea

Rescue

Ultra High Frequency (UHF)-(300 MHz-3 GHz)

Military Aircraft Band (319.1 - 383.9 MHz):

319.100: ............................................. FAA Traffic Control

321.000 - 336.600: ............................................. Air Force

342.500 - 344.600: ........................................... FAA Weather

346.400 - 364.200: ............................. Air Force Traffic Control

381.800 - 383.900: ........................................... Coast Guard

U.S. Government Band (406-420 MHz):

406.125 - 419.975: ............................................ GOVT, USXX

70-cm Amateur Band (420-450 MHz):

420.000 - 450.000: ................................................... HAM

Low Band (450-470 MHz):

450.050 - 450.925: ................................................... RTV

451.025 - 452.025: .................................. IND, OIL, TELM, UTIL

452.0375 - 453.00: ............................ IND, TAXI, TRAN, TOW, NEWS

453.0125 - 453.9875: ................................................. PUB

454.000: ............................................................. OIL

454.025 - 454.975: .................................................. TELB

455.050 - 455.925: ................................................... RTV

457.525 - 457.600: ................................................... BUS

458.025 - 458.175: ................................................... MED

460.0125 - 460.6375: ...................................... FIRE, POL, PUB

460.650 - 462.175: ................................................... BUS

462.1875 - 462.450: ............................................. BUS, IND

462.4625 - 462.525: ................................. IND, OIL, TELM, UTIL

462.550 - 462.725: ................................................... GMR

462.750 - 462.925: ................................................... BUS

462.9375 - 463.1875: ................................................. MED

463.200 - 467.925: ................................................... BUS

FM-TV Audio Broadcast, UHF Wide Band (470-512 MHz):

(Channels 14-69 in 6 MHz steps):

475.750: ...................................................... Channel 14

481.750: ...................................................... Channel 15

487.750: ...................................................... Channel 16

805.750: ...................................................... Channel 69

Note: Some cities use the 470-512 MHz band for land/mobile service.

Conventional Systems Band - Locally Assigned

851.0125 - 855.9875: ................................................. CSB

Conventional/Trunked Systems Band - Locally Assigned

856.0125 - 860.9875: ................................................ CTSB

Trunked Systems Band - Locally Assigned

861.0125 - 865.9875: ................................................. TSB

Public Safety Band - Locally Assigned

866.0125 - 868.9875: ................................................. PSB

Common Carrier

869.010 - 894.000: ................................................... CCA

Private Trunked

935.0125 - 939.9875: ................................................. PTR

General Trunked

940.0125 - 940.9875: ................................................. GTR

Frequency Conversion

The tuning of a station can be expressed in frequency (kHz or MHz) or in

wavelength (meters). The following information can help you make the

necessary conversions.

1 MHz (million)=1,000 kHz (thousand)

To convert MHz to kHz, multiply by 1,000:

9.62 MHz x 1000 = 9620 kHz

To convert from kHz to MHz, divide by 1,000

2780 kHz / 1000 = 2.780 MHz

To convert MHz to meters, divide 300 by the number of megahertz

300 / 7.1 MHz = 42.25 meters

A Note on Image Reception

Radios work by simple mathematics.  For example, most tune to a frequency

by mixing that frequency with another (local oscillator) frequency which

is slightly different. This mixing process primarily gives us the two

original frequencies, their sum, and their difference.  Well, the radio's

Intermediate Frequency (I.F.) filter normally passes either the sum or

difference frequency, and this is then processed into the sound we hear.

Because nothing is perfect, certain "harmonics" will also get through if

they are strong enough. For example, if a radio's I.F. is 10.7 MHz, we

might be able to tune to a frequency 21.4 MHz (2 x I.F.) above (or below,

depending on the radio's design) a strong signal and hear it! This is more

evident in a double-conversion radio than a triple-conversion radio,

because the triple-conversion radio's 1st intermediate frequency is quite

high.  This causes the image to be so far off frequency that it is easy to

effectively filter it out.

So remember that just because a radio doesn't receive something which

another does is not necessarily an indication of a problem.  The one radio

may simply not be "tricked" into picking up an image!  This rejection of

undesired signals is one reason that a triple-conversion receiver costs

more than a similar dual-conversion model.

You might be interested in finding more out about radio.  One good

location to start looking is your local public library. You might also

wish to contact the A.R.R.L., as they are an excellent source of

informative texts on the subject.

Contact:

Amateur Radio Relay League

225 Main St.,

Newington, CT 06111-1494

USA

(CD EB 2/22/00)

Privacy Policy