PRO-2006

(200-0145)                 Operation                  Faxback Doc. # 31350

PROGRAMMING THE PRO-2006 SCANNER

1.  Select a channel to program by pressing [MANUAL], and entering the

channel number you want to program.  Then, press [PROGRAM].  "PROGRAM"

appears in the display to indicate that your scanner is in the

programming mode.

2.  Enter a frequency.  Good references for active frequencies are Radio

Shack's "Police Call Guide Including Fire and Emergency

Services,""Official Aeronautical Frequency Directory", and "Maritime

Frequency Directory".  We update these directories every year, so be

sure to get a current copy.

3.  Press [ENTER] to store the frequency.  If you make a mistake in Step 2,

ERROR appears on the display and three beeps sound.  Press [CLEAR] and

repeat Step 2.

4.  Press [DELAY].  If you want your scanner to pause after each

transmission before scanning to the next channel, press [DELAY].

"DELAY" appears in the display.  If you do not want your scanner to

pause, press [DELAY] again.  "DELAY" disappears from the display.

5.  To program more channels, repeat Steps 1-4.  If you want to program

the next channel in sequence, just press [PROGRAM} and proceed to Step

2.

SEARCHING FOR AND TEMPORARILY STORING ACTIVE FREQUENCIES

If you do not have a reference to frequencies in your area, use these

procedures to search for a transmission.  See also "A Guide to the

Action Bands".

LIMIT SEARCH

The limit search procedure allows you to search within a range of

frequencies.  You can store up to ten limit search ranges in the search

banks.

1.  Press [PROGRAM], and the search bank number in which you wish to store

the limit search range.  Press [0] for bank number 10.

2.  Press [LIMIT].  "L" and the search bank number appears in the display.

3.  Enter the lower limit of the frequency range.  Press [ENTER].

4.  Press [LIMIT].  "L" changes to "H" in the display.

5.  Enter the upper limit of the frequency range.  Then, press [ENTER].

6.  Repeat steps 1-5 to store the limit search  range into the search

banks.  You can store up to 10 limit-search ranges.

7.  To recall a limit-search range, press the appropriate bank number.

8.  Press [UP-ARROW] to search from the upper limit down to the lower

limit.  Or, press [UP-ARROW] to search upward starting from the lower

limit.

9.  When the scanner stops on a transmission, press [MONITOR] to store the

frequency in the current monitor memory.  The memory number flashing on

the display indicates the current monitor memory.

10.  Press [UP-ARROW] or [DOWN-ARROW] to continue the search.

NOTES:  Press [SPEED] to speed up or slow down the search.

Press [DELAY] to make the scanner pause 2 seconds after a

transmission, before proceeding to the next frequency.

LIMIT SEARCH MEMORY

When you change your scanner's mode from limit search to manual, program,

scan, direct search, and so on, your scanner retains the last frequency

before you change the mode.  Your scanner resumes the search from that

frequency when you change the mode to limit search again.

NOTE:  You can change the upper or lower limit frequency without afecting

the memorized frequency if the frequency is within the new limit

range.  If the frequency is out of the new limit range, the search

starts from the new upper or lower limit frequency.

DIRECT SEARCH

1.  Select a currently programmed channel by pressing [MANUAL], and the

channel number.  Then, press either [MANUAL] or [PROGRAM].

2.  Press [DIRECT].  The step frequency appears in the display.

3.  Press [UP-ARROW] to search through higher frequencies or press [DOWN-

ARROW] to search through lower frequencies.

4.  When the search stops on a transmission, press [MONITOR] to store

that frequency in the current monitor memory.  The memory number

flashing on the display indicates the current memory number.  To

continue the search, press [DOWN-ARROW] or [UP-ARROW].

NOTES:  When you press [DIRECT] during limit, your scanner enters direct

search mode.

When you press a numeric key during a direct search, your scanner

changes to limit search mode.  The key you press corresponds with

the limit-search bank number.

MOVING A FREQUENCY FROM A MONITOR MEMORY TO A CHANNEL

As you store frequencies in monitor memories, the memory number flashing on

the display shows the current monitor memory.  You can listen to monitor

memories by press [MANUAL], [MONITOR], then the number of the monitor

memory you wish to listen to.

If there is a frequency you wish to transfer to a channel, follow these

steps to move it from a monitor memory

1.  Press [MANUAL].  Enter the channel number you want to store the

monitor frequency in; then press [PROGRAM].

2.  Press [MONITOR] and enter the monitor memory number that has the

frequency you want to move.

3.  Press [ENTER].  The scanner stores the monitor frequency in the

channel.

If you want to return to a limit search after this procedure, press

[LIMIT], and the search bank number.  Then, press either [UP-ARROW] or

[DOWN-ARROW] to continue searching.

USING THE RESTART SWITCH

The scanner's display might lock up the first time you plug in and turn on

your scanner, or if the battery is left out for an extended period of time.

If the display locks, use a pointed object, like a paper clip, to press and

release the restart switch while power is on.

To clear all the memories, be sure the scanner is turned on and:

1.  Press and hold [CLEAR].

2.  Using a pointed object, press and release the restart switch.

3.  After confirming that the display goes blank, release [CLEAR].

SETTING THE VOLUME AND SQUELCH

Follow these steps to set the volume and squelch.

1.  Turn VOLUME clockwise and SQUELCH counterclockwise until you hear a

hissing sound.

2.  Adjust VOLUME to a comfortable sound level.

3.  Slowly rotate SQUELCH clockwise until the hissing stops.

NOTE:  If the scanner picks up unwanted, partial, or very weak

transmissions, rotate SQUELCH clockwise to decrease the scanner's

sensitivity to these signals.

USING THE SOUND SQUELCH SWITCH

If the scanner stops at a transmission during scan, search, or priority

modes, the [SOUND SQUELCH] switch lets the scanner start scanning again if

the transmission contains no sound (carrier only without modulated

signals).

If you scanner stops at a frequency that has no sound, press [SOUND

SQUELCH].  The indicator lights.  If the scanner detects no sound within

0.5 seconds, it goes to the next transmission.

When the scanner receives a frequency that contains sound, it halts at the

frequency.  If the sound ceases, the scanner stays on the frequency for 5

seconds, and then resumes scanning.  If the carrier stops, the scanner

begins to scan immediately if the delay function is off, or after 2 seconds

if the delay function is on.

To cancel sound squelch, press [SOUND SQUELCH] again.  The indicator goes

off.

NOTE:  If a frequency contains a transmission with low modulation, the

sound squelch circuit might not work properly.

SCANNING THE CHANNELS

To begin scanning the channels, just press [SCAN].  Your scanner scans

through all the channels that you have not locked out of the banks that are

turned on.  Set SQUELCH so that you do not hear hissing sounds between

transmissions.

Be sure to read the following sections to get the full benefits from

all of your scanner's special features.

USING THE DELAY FEATURE

Many agencies use a two-way radio system that might have a pause of

several seconds between a query and a reply.  To program a delay into

that channel to keep from missing a reply, select the channel and press

[DELAY] so DELAY appears on the display.

When your scanner pauses at an active channel which has been programmed

with a delay, while scanning, it waits for 2 seconds after the completion

of each transmission on that channel before it resumes scanning.

Some radio systems, especially those above 800 MHz, use a special "trunked"

system.  In these systems, the transmitter selects an available frequency

each time the operator keys the radio.  Therefore, it is possible for the

query to be on one frequency and the reply on another.  To ensure the best

possibility of hearing the full reply, you want the scanner to begin

scanning immediately when the first transmission ends.  In this case,

select the channel manually and ensure that "DELAY" is not in the display.

If "DELAY" is indicated, press [DELAY] to turn off this feature for that

channel.

SETTING THE SCANNING SPEED

While you are in scan or search mode, you can press [SPEED] to select

the scanning speed.

The scanner has 2 different scanning speeds:

13 channels per second

26 channels per second

LOCKING OUT CHANNELS

You can increase the effective scanning speed by locking out specfic

channels that you have not yet programmed.  Manually select the empty

channel and press [L/OUT] so that "LOCK OUT" appears on the display.  This

is also handy for locking out channels you have programmed that have a

continuous transmission.  You can still manually select locked-out

channels.

To unlock a channel, manually select the channel and press [L/OUT],  Or,

press [L/OUT RVW] in manual or program mode to call out locked-out channels

one by one.  Then, press [L/OUT] again.

NOTE:  There must be at least one active channel in each bank.  You cannot

lock out all channels.

TURNING BANKS ON AND OFF

As explained in "Understanding Channel Storage Banks and Search Banks" your

scanner splits the 400 channels into ten banks of forty channels each.  The

small bars under the numbers at the top of the display are the bank

indicators.  You can turn each bank on and off, so that all the channels in

a bank are either scanned or locked out.  In scan mode, press the number

key for the bank you want to turn on or off.  If the bank indicator is on,

you have turned on the bank and the scanner scans all the channels within

that bank that you have not locked out.  If the indicator is off, the

scanner does not scan any of the channels within that bank.  You can still

manually select any channel in a bank, even if you have turned off the

bank.  You cannot turn off all banks - one must be turned on.

PRIORITY

You can scan through the programmed channels, and still not miss an

important or interesting call on a specific channel.  To program a stored

channel as the priority channel, press [PRI] during scanning.  The scanner

now checks the priority channel every two seconds, and stays on the channel

if there is any activity.

To turn on the priority feature, press [PROGRAM] and the desired channel

number.  Then press [PRI]. "P" appears on the display whenever the scanner

is set to the priority channel.  You can only program one channel as the

priority channel.  If you program a new channel as the priority channel,

the previous channel you chose is automatically cleared.

To turn off the priority feature, press [PRI] until "P" disappears from the

display.

NOTE:  Channel 1 is automatically designated as the priority channel the

first time you turn on your scanner.

MANUALLY SELECTING A CHANNEL

You can continuously monitor a specific channel without scanning.  This is

useful if you hear an emergency broadcast on a channel and do not want to

miss any details-even though there might be periods of silence-or if you

want to monitor only a locked-out channel.

Follow these steps to manually select a channel.

1.  Press MANUAL.

2.  Enter the channel number.

3.  Press MANUAL again.

Or, if the scanner is scanning and stops at the desired channel, press

MANUAL one time.  (Pressing MANUAL additional times causes the scanner to

step through the channels.)

DIMMER

Press [DIMMER] to turn the display's backlight down or up.

(03-19-96)

Privacy Policy